# Project title:

Simulation of Regenerative Braking System Using 3-Phase Induction Motor in MATLAB/Simulink

Student Details:

Ritvik Anish – 23BEE1018 (ritvik.anish2023@vitstudent.ac.in) 
Sanjh Yadav – 23BEE1043 (sanjh.yadav2023@vitstudent.ac.in)

Adithyan D Ajay – 23BEE1009 (adithyan.dajay2023@vitstudent.ac.in)

# Abstract:

This study is about simulating a regenerative braking process using a 3-phase induction motor in MATLAB/Simulink. It aims to demonstrate the process of converting the mechanical energy generated while braking into electrical energy instead of converting it into heat. Simulink is used for modeling in this project, where the motor will operate normally and then experience regenerative braking due to reduced frequency supplied to it. While the rotor rotates at a higher speed than the synchronous speed, the motor operates as a generator and converts mechanical energy back into electrical energy. Graphs representing the changes in speed, torque, and power of the motor are produced as an output from the simulation.

# Objectives:

- To simulate regenerative braking using a 3-phase induction motor

- To analyse motor performance during braking

- To observe power flow reversal

- To plot speed, torque, and power graphs

- To improve understanding of energy recovery systems

# Existing methods:

- Mechanical braking systems that dissipate energy as heat (losses)

- Dynamic braking uses resistors to absorb energy

- Plugging method reverses motor direction but wastes power

- Traditional systems do not focus on energy recovery

- MATLAB Simulink-based models on EVs, BLDC motors and single phase motors, not 3 phase induction motors

# Limitations of existing methods:

- Energy loss in the form of heat

- Low efficiency

- No energy regeneration

- Increased operational cost

- Limited analysis without simulation tools

- No existing software model that simulates

# Proposed methodology:

The proposed system is developed entirely using MATLAB Simulink.

A 3-phase induction motor model is created in Simulink. Initially, the motor operates under normal conditions with standard supply frequency. To simulate regenerative braking, the supply frequency is reduced suddenly. This causes the rotor speed to become higher than the synchronous speed, forcing the motor to operate in generating mode.

The system behaviour is analysed by observing:

- Speed variation

- Torque characteristics

- Power flow reversal

Graphs are plotted to verify regenerative action, where power changes from positive (motoring) to negative (generating).

Include  circuit diagram/ cad images if any

# Hardware components used (Simulink components):

| S. No | Component Name | Specification | Purpose |
| --- | --- | --- | --- |
| 1 | 3 phase ac supply | 400V peak | Input source voltage |
| 2 |  |  |  |

Implementation:

Describe how the project was implemented:

- Step-by-step procedure

- Software tools used (MATLAB, Python, Arduino, etc.)

- Experimental setup

- Data collection and processing

# Hardware Images:

Results and Discussion:

Present results using graphs, tables, or screenshots Compare with existing methods

# Novelty of your project:

Highlight what is new in your work

# Conclusion:

Summarize your project and outcomes

# References:

Is any dataset used for your project: Yes/No

If yes, Provide link to the dataset: drive link for own data collected

# Please indicate your willingness to extend this project work towards Patent Filing or Research Publication: Yes/ No

Guidelines

- Plagiarism of the document must be less than 15%

- Use Font- Times new roman, Size – 10 to 12 for content, Spacing – 1.15 or 1.5

- All figures and Tables must be numbered